From:         "John E. Williams" <xsdjew@IBMSRV.ORYX.COM>

Here is the current version of our HOW TO DO IT blurb on receiving

APT images from the NOAA and Soviet METEOR Weather Satellites.

John Williams

-------------------------   jewilliams@oryx.com   ----------------------

RECEIVING WEATHER SATELLITE IMAGERY: A BEGINNER'S PRIMER

--------------------------------------------------------

Courtesy Dallas Remote Imaging Group

Datalink RBBS ============> 214-394-7438

Dedicated to satellite tracking, decoding of NOAA/Soviet

meteorological satellite telemetry, and Digital Image

Processing of satellite pictures.

Jeff Wallach,      N5ITU, Chairman

John Williams,            Co-Chairman

John DuBois,       W1HDX, VAS/HRPT Design Engineer

T S  Kelso,               Air Force/NASA liaison

Ed O'Grady,               Soviet Space Program Analyst

Mark Sims,                Director Software Development

All aspects of Amateur Radio covered on Datalink RBBS

(IF YOU COPY THIS BULLETIN FOR OTHER SYSTEMS, PLEASE BE KIND ENOUGH

TO LEAVE  THIS BANNER  AS DUE CREDIT TO THE HARD-WORKING FOLKS WHO

DEVELOPED AND CONTINUALLY UPDATE THIS INFORMATIVE 'HOW-TO'BULLETIN)

Dayton HamVention  '91

(rev 14)

Tom  Gentry, K5VOU,  has  been  kind  enough  to  help  us prepare the

following:

HOW TO RECEIVE APT PICTURES FROM THE NOAA SATELLITES

----------------------------------------------------

So  you  have  decided you want to receive the NOAA and Russian METEOR

orbiters  and  you  have  a  radio  that  receives  137.500  MHz and a

computer.  So  now what do you do? I hope this short dissertation will

steer you in the correct direction.

To  display a picture from the orbiting birds you need several things.

First  you  need  to  'acquire' the satellite. This means you need the

program to calculate its position at any time and determine if you can

hear  the  signals from the 'bird'. The best place to get this program

is  probably AMSAT-NA an organization dedicated to AMateur SATellites.

They have several very good tracking programs for the IBM and also for

Commodore  and  Apple as well. After you have acquired the program and

have it running on your computer, you then need the Keplerian Elements

for  the  NOAA  satellites.  These  elements  as well as some tracking

programs  are  available from the CELESTIAL RCP/M, run by TS Kelso, at

513-427-0674 (Fairborn,OH) and from the Weather Fax and Remote Imaging

DATALINK BBS run by Dr. Jeff Wallach, N5ITU as bulletins for some 100+

sets of satellite  data.   Another bulletin also tells you to what the

printed  numbers  correspond  so  you can enter them into the computer

program.  Use  the  newest  elements  you  can  find  as  the orbiting

satellites are subject to forces which change their orbits. If you use

Keplerian Elements more than 3 or 4 weeks old, your chances of hearing

the bird are reduced.

Once you are tracking the birds you should hear them on your receiver.

They  will  be on 137.62(NOAA-9/11), 137.500(NOAA-10), and on 137.850,

137.400, and 137.300 for Soviet Meteorological Satellites/METEOR.  The

Chinese   satellite   Feng   Yun  transmits  on  137.795  when  it  is

operational.  Now comes the first thing different about NOAA orbiters.

The  frequency deviation of the FM transmission is about +/-18-20 kHz.

This  is  4  times  the  normal deviation of a police call and 1/3 the

deviation  of  the  FM  broadcast  band  FM signal. If you receive the

signal  on  the  regular communications width of the scanner the light

areas of the picture will be noisy and the signal will sound distorted

and the picture will basically be useless. If you use the FM broadcast

bandwidth  (assuming  your  scanner  will let you), the signal will be

weak and there may be too much background noise to see the picture.

So  what  now ?   Well  there  are two ways to fix the problem, find a

receiver  with  the  proper I.F. bandwidth filter of 50 kHz, or modify

the  I.F.  of  your scanner to about 50-80 kHz. The simplest mod which

has  been  found to be workable is to remove the narrow communications

I.F.  filter  and  replace it with a 0.01 uFd capacitor. This provides

for  good  Wefax  pictures  from  GOES  and  pretty  fair NOAA orbiter

pictures  -  see  March  1991,  '73 Amateur Radio Today' article for a

description of this procedure. It can of course result in retuning the

radio,  voiding the warranty and making the squelch not work properly.

The  other  characteristic of the NOAA satellite transmissions is THEY

ARE  WEAK.  To  get  good  pictures  over a large area we have found a

pre-amplifier  to  be  essential. A GaAs-FET type can be obtained from

various  sources at a reasonable price but still about 50-100 dollars.

You  can  also build one for about 25 dollars from plans for a pre-amp

for the 2-meter amateur band and tune it to 137.5 easily.

Lastly,  but  importantly,  the antenna can be of an omni directional,

uniform pattern type such as a turnstile antenna similar to those used

in  the  FM  broadcast reception business but of course tuned to 137.5

MHz. Mount the pre-amp at the antenna if possible. Crooked coathangers

on  broomsticks  have  been  known to work, and it is not difficult to

build the antenna. Of course 2-meter beams with Az/El tracking will do

the job well.

Now that you have a nice audio signal from the satellite, what happens

next? The audio tone of 2400 HZ which is the carrier tone that is used

to  carry  the  picture  information (video), must be detected and the

video  data converted from analog to digital and then displayed on the

computer monitor by the software. The hardware and software to do this

is  available  from  several  sources  with more coming along. Several

stand-alone  boxes  are  also  available  that  produce  some  form of

computer  output either in printed form or on the display. In addition

the  people  on the DATALINK BBS may also have some data to share with

you regarding this equipment.

One  of  the  best  sources  for  information is the WEATHER SATELLITE

HANDBOOK  by  Dr. Ralph Taggart. The 4th edition is now available from

the ARRL. A must is THE JOURNAL OF THE ENVIRONMENTAL SALELLITE AMATEUR

USERS'  GROUP - JESAUG - which is published quarterly. It is available

for  $30.00  per  year  from  the  editor, Jeff Wallach, PO Box 117088

Carrollton,  TX.  75011-7088.  Back  issues  of  the  JESAUG  are also

available.

One  good  place  to start is with the Elmer Schwittek MULTIFAX or the

MULTIFAX  MAP  software  which  runs on  the IBM PC.  The PC interface

hardware  is  available  as printed circuit board alone, board and all

parts,  or completely assembled.  Additional image processing software

is available online DATALINK.

The  units  available from David Schwittek, Ralph Taggart, Jerry Dahl,

GTI Electronics and Quorum communications listed under Sources are all

well worth investigating if you are just getting started.

You  can  obtain  directly  from  DATALINK  the MULTIFAX MAP interface

hardware designed by Jim Bartlett and David Schwittek.

'Typical' APT Receiving Station

-------------------------------

[1]   ANTENNA

-

[2]   PRE-AMP

-

-

-

-                          [ STAND ALONE UNITS ]

-

[3]   RECEIVER  ----------------->  FAX MACHINE  ---> PAPER

-                          WRASSE UNIT  ---> MONITOR

-                          YU3UMV UNIT  ---> MONITOR

- - - - VCR

-

-

-

[4] - VIDEO DETECTOR   -

-                  -

-                  -

[5] - ANALOG/DIGITAL   -

-                  -

-                  -            [ SCAN CONVERTERS ]

-                  -

[6] - INTERFACE TO PC  - <=======>  ROBOT 1200

-                  -            TAGGART'S 1700

-                  -            IMAGEWISE

-                  -

[7] -    SOFTWARE      -            ** SECOND DISPLAY **

-                  -               for weather images

-                  -

-                  -

[8] - GRAPHICS ADAPTOR -

-

-

-

-

[9]   PC DISPLAY / MONITOR

Display weather images

(or commands for SCAN CONVERTERS)

-

-

[10]   PC PRINTER / DOT MATRIX / LASER

NOTES:

------

[4] VIDEO DETECTOR, [5] ANALOG/DIGITAL CONVERTER, and

[6] INTERFACE TO PC may be in separate hardware units

or all on one circuit board.

Sources:

----------------------------------------------------------------------

NOAA Data / Keplerian Elements:

DATALINK RBBS, N5ITU, Dr. Jeff Wallach, 214-394-7438,

DALLAS REMOTE IMAGING GROUP ELECTRONIC BULLETIN BOARD

UPDATES ON NOAA BULLETINS, SATELLITE LAUNCHES, FREQUENCIES

DISPLAY PICTURES FROM NOAA SATELLITES ON YOUR OWN P.C.

CELESTIAL RCP/M, TS Kelso, 513-427-0674, NASA Prediction Bulletins

are online and updated several times weekly

Canadian Space Society BBS, 416-458-5907,  2-Line Elements  maintained    |

by Ted Molczan of 600+ satellite - Visual satellite tracking info        |

Satellite Tracking Software:

AMSAT-NA,  PO  Box  27, Washington, DC 20044, Software Exchange,

phone 301-589-6062 (voice), N4HY - QUIKTRAK, W0SL - ORBITS II,III,IV,

and InstantTrack by Franklin Antonio.

Roy D. Welch, W0SL, 908 Dutch Mill Drive, Manchester, MO 63011,

phone 314-391-1127, ORBITS II-CGA, ORBITS III-EGA, ORBITS IV-VGA

DATALINK BBS, download PASSCHED.ZIP, SEESATxx.ARC, TRKSAT.xxx and others

CELESTIAL RCP/M, TS Kelso, 513-427-0674, PASSCHED.ZIP or SEESATxx.ARC

Astronomical Data Service, Rodger Mansfield, 3922 Leisure Lane,

Colorado Springs, CO 80917-SPACE BIRDS program and Newsletter, $40.

Paul E. Traufler, 111 Emerald Drive, Harvest,AL 35749-TRAKSAT Satellite   |

Tracking program-Non - Registered $10.00, Registered Version $25.00.     |

Bill Bard, 1732 74th Circle NE, St Petersburg, FL 33702, MACSAT           |

tracking program for the Apple Macintosh - $10.00 for disk and manual.   |

RPV Astronomy BBS, Dave Ransom, 213-541-7299, STSORBIT Tracking program   |

and latest version of TRAKSAT and NASA Prediction Bulletins online.      |

Receivers:

Vanguard Labs, 196-23 Jamaica, Hollis, NY 11423,

phone 718-468-2720, BBS Dataline 718-740-3911

Hamtroincs, Inc.,65-D Moul Road, Hilton, NY 14468-9535

phone 716-392-9430

DARTCOMM, Mr. N. Hearn, DARTCOMM Ferndale, Postbridge, Yelverton,

Devon PL20 6SY, Great Britain, UK, phone 011 44 0822 88253

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

phone 817-488-4861, BBS Dataline 817-421-0228                            |

Scanner Modification, See March, 1991 '73 Amateur Radio' Magazine for     |

'Weather Satellite Reception' article by John E. Hoot, page 12.          |

Crystals:

JAN  Crystals,  2400  Crystal  Dr,  Fort  Myers,  FL 33906-6017,

phone 1-800-237-3063 or 813-936-2397

Pre-Amps:

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

Spectrum   International,Inc.,  Box  1084,  Concord  MA  01742,

phone 508-263-2145

Vanguard Labs, Hamtronics and GLB for Helical RF Filters for intermod

Ehrler-DuBois Very High Performance LNA for 1500-1750 Mhz, contact

Greg Ehrler or John DuBois via DATALINK

Antennas:

ARRL Handbook, 1986, turnstile over ground antenna.

ARRL Satellite Experimenter's Handbook by Martin Davidoff, K2UBC

Modern Electronics, September 1988, Hank Brandli article, page 82,

Receiving Satellite Weather Photos

Hardware Interface to PC:

A&A  Engineering,  2521  W. La Palma, Unit K, Anaheim, CA 92801,

phone 714-952-2114 - both AM dectector and A/D converter, input to PC

via game port (4-bits) or parallel I/O Interface (8-bits) - kit is

$50. - assembled and tested unit is $70.

Microcomm,  H. Paul Shuch, N6TX, 14908 Sandy Lane, San Jose, CA 95124,

phone 408-377-6137 - FAXBOARD - A Weather Facsimile Display Board for

the  IBM  PC  -  Complete  documentation  to build your own interface

circuit  on  an  IBM  PC  prototyping  board - video and an 8-bit A/D

interface - $6.00, also in QEX, Sep '88.

Video Detectors:

RTM  Circuit  Boards,  205  Elm  St.,Van  Horne, IA  52346-0400,

video detector  (the 'Wilson Board')  see QST Magazine, Jan '86 and

Vince Coppola's  article in Oct 1988  73 Magazine

QST Magazine, August 1985, Grant Zehr article, page 27, video detector

circuit

A/D Converter & Parallel I/O Interfaces for the IBM PC:

MetraByte,  440  Myles Standish Blvd., Taunton, MA 02780, Model PIO-12

board for the IBM PC, phone 617-880-3000

John  Bell  Engineering, Inc, 400 Oxford Way, Beimont, CA 94002,

phone 415-592-8411

Sources:

----------------------------------------------------------------------

Display Software and Hardware - 'Amateur':

Multifax,  David  Schwittek,  1659 Waterford Road, Walworth, NY 14568,

phone  315-986-2719, Multifax 5.x and 4.x for VGA, 3.x for EGA on the

IBM-PC. This is the original 16 color system, see Multifax MFMAP.

Clay Abrams Software, 1758 Comstock Lane, San Jose, CA 95124, software

for the ROBOT 1200 - see QST Magazine, Jan '86

Ralph  Taggart,  602  S. Jefferson, Mason, MI 48854, Weather Satellite

Handbook  Scan  Converter - New Model 1700 APT Scan Converter for the

the  IBM PC. Order Handbook from the ARRL, Scan Converter from Metsat

Products,  1257  Glen  Meadows  Ln,  East  Lansing  MI,  48823, phone

517-332-7665.   Also   see   73   Magazine,  Nov  '84  and  Dec  '84,

WSH/Handbook,  and  WEATHERSAT  columns  in 73 Magazine from 1987 and

1988.  Weather Satellite Handbook BBS, 517-676-0368, Ralph Taggart.      |

ImageWise  Display/Receiver,  Circuit  Cellar Inc.  -  Micromint Inc.,

4 Park Street,  Vernon, CT 06066, phone 203-875-2751 - BYTE Magazine,

May,  Jun,  Jul,  Aug 1987. JESAUG 88-2 and Oct 1988 73 Magazine have

information  on  how  to  build  the video and A/D interface. Ver 2.0

software  supports  VGA  graphics and 640 samples per line at 64 gray

levels.  With  VGA  the  ImageWise  Display unit is not required. The

software  is  available for $29.95 from Vince Coppola, N1VC, 6 Bobbin

Road, Terryville, CT 06786

ASAT:  An Apple-based Satellite Imaging System by Grant Zehr, WA9TFB -

QEX  Magazine,  March  '88  -  has  complete  information to build an

excellent  unit  using  the  APPLE  with a Redshift Ltd graphics card

giving 256 * 256 pixels at a full 256 levels of gray.

AMIGA Imaging Software - Dallas Remote Imaging Group

PO Box 117088

Carrollton, TX 75011-7088

Voice 214 394 7325

Uses standard Amiga hardware - no special display

Powerful NASA digital enhancement curves built-in

Digital image processing capabilities

Uses A&A Engineering $49 demodulator

Image saved for use by other graphics programs

Full histogram analysis

Full 8 bit data is stored on diskette or hard disk

Unattended operation

Can be setup in 30 minutes

Call for special pricing

Multifax MFMAP, Elmer and David Schwittek, 1659 Waterford Road,           |

Walworth, NY 14568, phone 315-986-2719. MFMAP software for the IBM PC    |

and  the  Jim Bartlett/David Schwittek/DATALINK  interface  card  for    |

APT/GOES/FAX.  This  is  a fully functional 8 -bit / 256 color system    |

with image data captured directly to disk at 4800 or 3600 samples per    |

second.  Additional software is available online DATALINK and Kelso's    |

CELESTIAL BBS - download APTCAPxx.ZIP and IMDISPxx.ZIP.                  |

A Weather-Facsimile Package for the IBM PC  by Jerry Dahl in April and

May  '90  QST describes how to construct a complete FAX system. Notes

at  the  end  of  article  contain  an excellent list of hardware and

software  sources.  The  completed  card can be ordered directly from    |

Jerry  Dahl,  OFS  Software,  6404 Lakerest Court, Raleigh, NC 27612,    |

phone 919-847-4545                                                       |

WeatherFAX-GTI Electronics, 1541 Fritz Valley Rd, Lehighton, PA 18235,    |

has  a  complete  line  of  satellite receiving equipment,               |

George Isleib,  phone 717-386-4032                                       |

Complete Systems - Hardware and Software:

Dallas Remote Imaging Group

P.O. Box 117088

Carrollton, Texas 75011-7088

214 394 7325

Complete Earth-Scan AMIGA turnkey system ready to put on air

YU3UMV  Digital  Storage  and  Scan  Converter  for  Weather Satellite

Images,  described  in  VHF  Communications Magazine, Winter 4/82 and

Spring  1/83  -  Modifications  are described in the Satellite Users'

Group  Journal - JESAUG - available from Jeff Wallach. Back issues of

VHF   Communications  and  the  YU3UMV  printed  circuit  boards  are

available from Fred Sharp

Loren  Johnson,  P.O.  Box  219,  Cleveland,  MN 56017, system for the

IBM-PC  -  $750,  phone  507-931-4849

WEATHERTRAC[tm] IBM-PC Based Satellite Image Acquisition and Animation

System  -  $1885  for  EGA - $3495 for VGA, Fred Bartlett, Softworks,

Inc,  Allentown,  PA, phone 215-395-4441 or George Isleib, Lehighton,

PA, phone 717-386-4032

Quorum Communications, 1020 S. Main St. Suite A, Grapevine, TX 76051,

phone   817-488-4861,   BBS Dataline 817-421-0228,  offers  a  fully

integrated  systems for  the  IBM  PC  including GOES down converter,

GOES/APT receiver,  Wefax  PC  Adapter  and software. A demo disk is

available  on  request or via the Quorum BBS. Quorum also sells the     |

AVHRR/HRPT system designed by John DuBois and Ed Murashie.              |

Spectrum International,Inc.,  Box  1084,  Concord  MA  01742, phone      |

508-263-2145, is the U.S. representative for Timestep Weather Systems   |

SSC/Software Systems Consulting, 150 Avenida Cabrillo, Suite C,          |

San Clemente, CA 92672 - PC HF Facsimile and PC GOES/WEFAX software     |

and complete receiving systems, phone 714-498-5784                      |

Satellite Meteorology:

Satellite  Imagery  Interpretation  for Forecasters  -  a 3 volume set

published  in  1985  for NOAA now available from the National Weather

Association,  4400  Stamp  Road,  Room  404,  Temple Hills, MD 20748,

301-899-3784,  $32  for  members, $45 for non-members ( Membership is

$20. )

Bibliography - Online DATALINK Bulletins:   (214 394 7438 -- BBS)

NASA Satellite Prediction Bulletins, NASA-PB.xxx

How to use NASA 2-Line Keplerian Elements

AMSAT Bulletins and Weekly Notes / Orbital Elements

NOAA APT NOTES

NOAA User Bibliography

Summary of Soviet Meteorological Satellites by Grant Zehr

How to Build a Weather Satellite Groundstation (This Primer)

Wrasse Dedicated System Review

GOES VAS Imaging System for the IBM PC, John DuBois W1HDX

LNA for WEFAX-VAS-HRPT-GPS, John Dubois and Greg Ehrler

GOES I-M Conference Summary

Ralph Taggart's New  APT Scan Converter and Book

Dartcom 137-138 Synthesized Receiver Packaging, John DuBois

Amateur Radio News from Newsline, Bill Pasternack

How to VISUALLY OBSERVE the Space Shuttle, Ted Molczan

AMSAT InstantTrack V1.00 program description, Franklin Antonio

Review DATALINK RBBS bulletins for current information ====> 214 394-7438

DALLAS REMOTE IMAGING GROUP - DRIG  Voice: 214 394-7325

Data: 214 394-7438

P.O. Box 117088

Carrollton, TX 75011-7088

ATTENTION:  Jeff Wallach, Chairman

|

New or Changed Items = |

|